  Prof. H. K. D. H. Bhadeshia,

University of Cambridge, December, 1999

http://www.msm.cam.ac.uk/map/mapmain.html

This file contains the steps to run the Model.

1) Copy files onto hard disk, without disturbing the structure and remove the "READ-ONLY" status for files 'test.dat' and 'Result.dat'.

2) Edit the test.dat file to set input variables. The input variables are in row;

Column	Variables

1	 Temperature (deg. C)

2	 Strain Rate (per second)

3	 Interpass Time (s)

4	 C     (wt%)

5	 Si    (wt%)

6	 Mn  (wt%)

7	 Cr   (wt%)

8	 Ni    (wt%)

9	 Nb   (wt%)

10	 V     (wt%)

11	 Ti    (ppm. wt.)

12	 Strain

13	Measurement History (largest strain value, of the previous stress-strain measurement)

3) Double click on "model.exe” icon , to run program.

4) Finally, the results are written in "Result.dat". The format of output (Stress);

Predicted  	Error   Predicted–Error  Predicted+Error

(MPa)    (MPa)        (MPa)            (MPa)

End of task.