4.3  HAZ Hardness Module

The HAZ hardness module computes the 800 to 500oC cooling time (sec)

and the heat-affected zone (HAZ) hardness values from eight independent weld

hardening models available for this purpose (see background material section).

-	select 'HAZ HARDNESS' from the main interface.

The HAZ hardness module interface comes up to the screen in a few

seconds, as shown in Figure 4.10. The menu bar is on the top of the screen. The

right hand column is the input data editor. The lower left hand box is the output

results window which contains hardness values (in VHN) and fraction of

martensite (in % martensite) values calculated from two of the models. The 800 to

500oC cooling time (t8/5 in sec) is also displayed. At the center of the screen is the

module name - HAZ Hardness Module and the regression limits status is also

shown.

Entering Welding Parameters

The upper section of the input data editor is reserved for welding

conditions and the lower section is base metal chemical composition. The welding

conditions are necessary to calculate the cooling time t8/5. It can be replaced by an

experimental cooling time directly submitted to the system if it is available. The

following data is an example:

-		type :-	300 	<enter>	{welding current}

25  	<enter>	{welding voltage}

5	<enter>	{amb. temperature}

50	<enter>	{plate thickness}

5	<enter>	{welding speed}

90	<enter>	{process efficiency}

.12	<enter>	{C}

1.29	<enter>	{Mn}

.26	<enter>	{Si}

.15	<enter>	{Ni}

.15	<enter>	{Cr}

.05	<enter>	{Mo}

.1	<enter>	{V}

0	<enter>	{B}

.01	<enter>	{Cu}

.01	<enter>	{Nb}

.01	<enter>	{N}

.01	<enter>	{S}

You have now entered all the necessary data (both welding conditions and steel

chemical composition) and the calculation command can be activated - 'F1

CALC'.

-	select 'F1 CALC'.

The output window is updated with the calculated HAZ hardness values,

two fraction martensite values and cooling time t8/5, as shown in Figure 4.11. The

maximum calculated, minimum calculated and average calculated HAZ hardness

values are also presented in a pop-up window. 'N/A' stands for 'not applicable'

which arises when the input parameter(s) is/are beyond the regression limits and

the system regression limits function is activated.

You may want to input the experimental cooling time t8/5 instead of calculating it

from the welding conditions.

-	select 'F2 T8/5' from the menu bar.

The first section of the input data editor (welding conditions) is changed to ' T

(8/5)', as shown in Figure 4.12. You can type in your own cooling time t8/5.

-	type :-  10	<enter>.

You have entered 10 sec for t8/5.

-	select 'F1 CLAC'.

The system responds the same as it would were you using the welding conditions:

eight different HAZ hardness values together with calculated maximum,

calculated minimum and calculated average hardness values, two fraction

martensite values and cooling time t8/5, as shown in Figure 4.12.

-	select 'F3 EDIT'.

You are now returned to use the welding conditions for determining t8/5. You are

free to select between these two choices.

More Information about the Regression Limits

-	select 'F4 LIMI' from the menu bar.

A pop-up menu appears on which the regression limits of different models

are listed, as shown in Figure 4.13.

Plotting a Graph (using welding condition)

-	select 'F6 PLOT' from the menu bar.

A pull-down menu appears as shown in Figure 4.14. This menu allows

you to select the x-axis title which is one of the input welding parameters. For

example :

-	select 'Mn'.

Another pull-down menu appears, as shown in Figure 4.15, which gives you the

choice of the y-axis title (one of eleven computed results : eight models, two

fractions  martensite values or the cooling time t8/5). For example:

-	select 'Suzuki'.

Pop-up windows appear which ask you to enter the 'lower limit' and the 'upper

limit'. For example:

-		type :-	.5	<enter>	{lower limit}

2.0	<enter>	{upper limit}

You have now asked the system to generate a graph of Suzuki calculated

HAZ hardness vs Mn content from 0.5 to 2.0 wt%, as shown in Figure 4.16.

Plotting the Characteristic Hardening Curve

The so-called characteristic hardening curve is a plot of HAZ hardness

against cooling time over a wide range of values, typically 1s to 100s for most low

carbon steels. Usually an x-axis log scale is used for cooling time. To do this,

-	select 'F2 T8/5'.

-	select 'F6 PLOT'.

A pull-down menu appears which allows you to choose the y-axis title

from eight different hardness models. The x-axis title is always cooling time.

-	select 'Suzuki'.

Pop-up windows appear and ask for minimum and maximum cooling time.

-		type :-	1	<enter> {minimum}

100	<enter> {maximum}

You have just asked the system to plot a Suzuki calculated characteristic hardness

curve (CHC - HAZ hardness vs t8/5 cooling time) with the cooling time t8/5 ranging

from 1 sec to 100 sec on a log scale, see Figure 4.17. The maximum and

minimum cooling times are confined to 100 sec and 0.1 sec respectively for

plotting.

Data File

With this feature, you can save the current data to a file, retrieve data from

a file, delete a system data file or edit a existing system data file.

-	select 'F7 FILE' from the menu bar.

A pull-down menu appears as shown in Figure 4.18. Two types of data

files are available : welding conditions (*.HAZ) and steel chemical compositions

(*.STE). Steel chemical composition data files are shared with other modules

(reheat hardness and preheat treatment). For example:

-	select 'PROCESS' from the pull-down menu.

Another pull-down menu appears after this selection which contains choices of

'RETRIEVE', 'ADD', 'EDIT', 'DELETE' or 'EXIT'. Selecting 'EXIT' will end this

feature. For example:

-	select 'ADD'.

You are asked to enter the file name of the process data file you are going

to create. For example:

-	type :-   DATA1  <enter>

The current welding conditions are echoed in the pop-up window. You

can make changes before you save them to the file 'DATA1.HAZ'. After changes

have been made,

-	press <ESC>

to save data from the pop-up window to the file.

If you want to edit a system data file,

-	select 'F7 FILE'.

-	select 'PROCESS'.

-	select 'EDIT'.

A list of available files appears on the pull-down menu.

-	select 'DATA1'.

You are asked to enter a new file name or press <enter> if the existing file

name is not going to be changed. Then the welding conditions in the file 'DATA1'

are retrieved to the pop-up window. After you have finished editing,

-	press <ESC> to save the data.

If you want to delete a system data file,

-	select 'F7 FILE'.

-	select 'PROCESS'.

-	select 'DELETE'.

A list of welding conditions data files appears on the pull-down menu.

-	select 'DATA1'.

A pop-up window appears and asks for confirmation to delete.

-	type :- 'y' to confirm the deletion.

Actually the file is not deleted; the extension of it is changed to *.BAK and it

becomes invisible to the system. It can be re-activated later by changing the

extension of the DOS command if desired.

If you want to delete the file permanently:

-	type :-  del *.bak {at the DOS prompt}.

Welding parameters can be retrieved from data files directly instead of

typing in from the keyboard.

-	select 'F7 FILE'.

-	select 'PROCESS'.

-	select 'RETRIEVE'.

-	select a process from the listing (e.g. 'WELD1').

The input data are automatically updated to the input data editor.

Steel chemical compositions can be manipulated in the same manner, e.g.

-	select 'F7 FILE'.

-	select 'STEEL'.

-	select 'RETRIEVE'.

-	select 'B3460G50'.

Output to a Printer

-	select 'F8 PRIN', and follow the pop-up window instructions.

For further instructions, please refer to the 'Quick Tutorial' section 'Output to a

Printer'.

On-Line Help

-	select 'F9 HELP'.

A pop-up window appears as shown in Figure 4.19. It can be removed by pressing

any key or the mouse button.

Exiting the HAZ Hardness Module

-	select <ESC> from the menu bar with the mouse or,

-	press <ESC>.

The system will put you back to the main interface.

4.4  Reheat Hardness Module

In the reheat hardness module, the 800 to 500oC cooling time (sec) and the

as-welded and reheated HAZ hardness values (see background material section)

are calculated. For example:

-	select 'REHEAT HARDNESS' from the main interface.

The reheat hardness module interface appears as shown in Figure 4.20. The menu

bar is on the top of the screen. The right hand column is the input data editor. The

lower left hand corner is the output results window which contains the as-welded

and the post weld heat treated hardness values (in VHN) and the 800 to 500oC

cooling time (t8/5 in sec). At the center of the screen is the module name - Reheat

Hardness Module, and it also indicates the regression limits status.

Entering Welding Parameters

The first section of the input data editor is reserved for the welding

conditions. The middle part of the input data editor is the reheat treatment

parameters, followed by the base metal chemical compositions in the lower

section. The welding conditions are used to calculate the cooling time t8/5 which

can be replaced with an experimental cooling time directly input into the system if

so desired. For example, start entering data as follows:

-		type :-	300 	<enter>	{welding current}

25  	<enter>	{welding voltage}

5	<enter>	{amb. temperature}

50	<enter>	{plate thickness}

5	<enter>	{welding speed}

90	<enter>	{process efficiency}

620 	<enter>	{reheat temperature}

2	<enter>	{reheat time}

.12	<enter>	{C}

1.29	<enter>	{Mn}

.26	<enter>	{Si}

.15	<enter>	{Ni}

.15	<enter>	{Cr}

.05	<enter>	{Mo}

.1	<enter>	{V}

0	<enter>	{B}

.01	<enter>	{Cu}

.01	<enter>	{Nb}

You have already entered all the necessary parameters (welding conditions, reheat

parameters and steel chemical composition) and now you can activate the

calculations command - 'F1 CALC'.

-	select 'F1 CALC'.

The output window is updated with the calculated t8/5, as-welded and post

weld heat treated HAZ hardness values, as shown in Figure 4.21. 'N/A' stands for

'not applicable which arises when the input parameter(s) is/are beyond the

regression limits.

You may want to input an experimental cooling time t8/5 instead of

calculating it from the welding conditions.

-	select 'F2 T8/5' from the menu bar.

The first section of the input data editor (welding conditions) is changed to 'T

(8/5)'. You can type in your own weld cooling time t8/5. For example:

-	type :-	10 <enter>.

You have entered 10 sec for t8/5.

-	select 'F1 CLAC'.

The system responds the same as it would if the welding conditions were input.

-	select 'F3 EDIT'.

You have now returned to the welding condition mode for determining the

cooling t8/5. You are free to select from among these two choices.

Plotting a Graph (using welding conditions)

-	select 'F6 PLOT' from the menu bar.

A pull-down menu appears as shown in Figure 4.22. This menu allows

you to select the x-axis title which is one of the input welding parameters (either a

welding parameter, a reheat parameter or  a steel chemical composition). For

example:

-	select 'Mn'.

A pop-up window appears which asks you to enter the 'lower limit' and then the

'upper limit'. For example:

-  	type :  -.5	<enter>

2.0	<enter>

You have asked the system to generate a graph of hardness values vs Mn

content from 0.5 to 2.0 wt%, as shown in Figure 4.23. Two curves are generated

by the system. The darker curve represents the post weld heat treated HAZ

hardness values and the lighter curve represents the as-welded HAZ hardness

values. However, if one of the reheat parameters is selected as the x-axis title, only

the post weld heat treated hardness values are plotted since the as-welded hardness

will not be affected by the reheat parameters.

Plotting a Graph (using experimental cooling time t8/5)

If you are using experimental cooling times, both as-welded and post weld

heat treated hardness values are plotted against the cooling time on a log scale. For

example:

-	select 'F2 T8/5'.

-	select 'F6 PLOT'.

Pop-up windows appear and ask for minimum and maximum cooling times.

-		type :-	1	<enter>	{minimum}

100	<enter>	{maximum}

You have just asked the system to plot a characteristic hardness curve and a

corresponding reheat hardness curve (CHC - HAZ hardness vs t8/5 cooling time)

ranging from 1 sec to 100 sec on a log scale, see Figure 4.24.

Data File

With this feature, you can save the current data to a file, retrieve data from

a file, delete a system data file or edit a existing system data file.

Two types of data files are available : welding processes (*.REH) and steel

chemical compositions (*.STE). The welding process files contain both the

welding processes and reheat parameters. Steel chemical composition data files

are shared with other modules (HAZ hardness and preheat treatment).

Note : For further information on this part, please refer to the section 'Data File'

contained in section 4.3 'HAZ Hardness Module'.

Output to a Printer

-	select 'F8 PRIN', and follow the pop-up window instructions.

For further instructions, please refer to the 'Quick Tutorial' section 'Output to a

Printer'.

On-Line Help

-	select 'F9 HELP'.

A pop-up window appears as shown in Figure 4.25. It can be removed by pressing

any key or the mouse button.

Exiting the Reheat Hardness Module

-	select <ESC> from the menu bar with the mouse or,

-	press <ESC>.

The system will put you back to the main interface.

4.5  Preheat Treatment Module

The preheat treatment module determines the critical 1500 to 100oC

cooling time (sec) and from that a preheat temperature to avoid underbead cold

cracking (see background material section).

-	select 'PREHEAT TREATMENT' from the main interface.

The preheat treatment module interface replaces the main interface, as

shown in Figure 4.26. The menu bar is on the top of the screen. The right hand

column is the input data editor. The lower left hand corner is the output results

window which holds the critical 1500 to 100oC cooling time (t15/1 in sec), the

preheat temperature (in oC) and the preheat time (in sec) if a local treatment is to

be used. At the center of the screen is the module name - Preheat Treatment

Module, and it also indicates the regression limits status.

Entering Welding Parameters

The first section of the input data editor is the welding conditions. The

middle section is the joint geometry and restraint parameters, followed by the base

metal chemical compositions in the lower section. The preheat method and also

relevant parameters are located in the lower center part of the screen. For example,

start entering data as follow:

-		type :-	300 	<enter>	{welding current}

25  	<enter>	{welding voltage}

5	<enter>	{amb. temperature}

50	<enter>	{plate thickness}

5	<enter>	{welding speed}

90	<enter>	{process efficiency}

15 	<enter>	{hydrogen level}

600	<enter>	{rf}

500	<enter>	{yield strength}

3	<enter>	{Kt}

.12	<enter>	{C}

1.29	<enter>	{Mn}

.26	<enter>	{Si}

.15	<enter>	{Ni}

.15	<enter>	{Cr}

.05	<enter>	{Mo}

.1	<enter>	{V}

0	<enter>	{B}

.01	<enter>	{Cu}

.01	<enter>	{Nb}

-	select 'F2 UNIF'.		{select uniform preheat}

A pop-up window asks for the workpiece length and width.

-		type :-	100	<enter>	{length}

100	<enter>	{width}

-	press <ESC> to confirm the dimensions.

You have already entered all the necessary parameters (welding conditions,

restraint parameters, uniform preheat, workpiece dimensions and steel chemical

composition) and now you can activate the calculation command - 'F1 CALC'.

-	select 'F1 CALC'.

The output window is updated with the calculated cooling time t15/1 and

preheat temperature, as shown in Figure 4.27. 'N/A' stands for 'not applicable' (the

preheat time is not applicable for uniform reheat).

You may want to select local preheat instead of uniform preheat.

-	select 'F3 LOCA' from the menu bar.

A pop-up window appears and asks for heater input heat intensity and heater

width.

-	type :-	.1 <enter>

10 	<enter>

-	press <ESC> to confirm the input data.

You have selected local preheat and set the heater heat input intensity and heat

width to 0.1 J/(mm2 s) and 10 mm respectively.

-	select 'F1 CLAC'.

The system may respond more slowly when using local preheat. This is normal

because local preheat requires more iteration. The output data include cooling time

t15/1, preheat temperature and preheat time, as shown in Figure 4.28.

Furthermore, the system also includes typical heat intensity values for

electrical strip heaters and torches, <F4 STRI> and <F5 TORC> respectively (see

Chapter 5.3 for details).

Plotting a Graph (using local preheat)

-	select 'F6 PLOT' from the menu bar.

A pull-down menu appears as shown in Figure 4.29. This menu allows

you to select the x-axis title which is one of the welding parameters or a steel

chemical composition. The welding parameters are current, voltage, ambient

temperature, piece thickness, welding speed, process efficiency, the hydrogen

level, the restraint parameters: restraint coefficient rf, yield strength & stress

concentration factor Kt, and the local preheat parameters: heater intensity and

heater width. For example:

-	select 'PROCESS'.

-	select 'CURRENT' from the second pull-down menu.

Then the system asks for the y-axis title which should be one of T15/1, preheat

temperature or preheat time for local preheat,

e.g. :

-	select 'PRE TEMP' from the pull-down menu.

Pop-up windows appear which ask you to enter the 'lower limit' and the 'upper

limit', e.g. :

-		type :-  	200	<enter>	{lower limit}

500	<enter>	{upper limit}

You have now asked the system to generate a graph with preheat

temperature vs welding current from 200 to 500 amps, as shown in Figure 4.30.

Plotting a Graph (using uniform preheat)

-	select 'F2 UNIF'.

-	press <ESC> to confirm the workpiece dimensions.

-	select 'F6 PLOT'.

-	select 'PROCESS'.

-	select 'CURRENT' from the second pull-down menu.

Then the system asks for the y-axis title which should be t15/1 cooling time or,

preheat temperature for uniform preheat.

-	select 'PRE TEMP' from the pull-down menu.

Pop-up windows appear which ask you to enter the 'lower limit' and the 'upper

limit'. For example:

-		type :-	200	<enter>

500	<enter>

You have asked the system to generate a graph of preheat temperature vs

welding current from 200 to 500 amps, as shown in Figure 4.31.

Data File

With this feature, you can save the current data to a file, retrieve data from

a file, delete or edit a existing system data file.

Two types of data files are available : welding processes (*.PRE) and steel

chemical compositions (*.STE). The welding process files contain the welding

process parameters, restraint parameters, preheat method and preheat parameters.

Steel chemical composition data files are shared with other modules (HAZ

hardness and reheat hardness). For example:

-	select 'F7 FILE'.

-	select 'PROCESS'.

-	select 'ADD'.

-	type :-	DATA1 <enter>

You have just entered the file name of your new data file.

The current process data are echoed in the pop-up window, as shown in

Figure 4.32. They can be edited before saving.

-	press <↓> until 'LOCAL PREHEAT' has been reached (highlighted).

'LOCAL PREHEAT' can be changed to "UNIFORM PREHEAT' by pressing

<←> or <→>. Note that when the preheating method is changed, the input data

titles below it also change, see Figure 4.33. For example:

-	select 'LOCAL PREHEAT' by pressing <←> or <→>.

-	press <ESC> to save the data.

Note : For further information of this part, please refer to the section 'Data File'

contained in 'HAZ Hardness Module'.

Output to a Printer

-	select 'F8 PRIN', and follow the pop-up window instructions.

For further instructions, please refer to the 'Quick Tutorial' section 'Output to a

Printer'.

On-Line Help

-	select 'F9 HELP'.

A pop-up window appears as shown in Figure 4.34. It can be removed by pressing

any key or with the mouse button.

Exiting Reheat Hardness Module

-	select <ESC> from the menu bar with the mouse or,

-	press <ESC>.

The system will put you back to the main interface.