WELDSOFT 2

Software for Welding Engineers

written by Billy Chan & Malcolm Bibby

Aug. 1989 - 1995,  Canada.

Department of Mechanical & Aerospace Engineering,

Carleton University, Ottawa, Ont. K1S 5B6.

LIMITED WARRANTY

There is no warranty whatsoever. However, the software is faithful to the

mathematical relationships of the original welding process models (see references

in each section). Faith in this software must not preclude experimental

certification. However, it does provide guidance and no doubt limits the extent of

trial and error experimental programmes which are often necessary to arrive at

optimum welding conditions.

LICENSE AGREEMENT

Copyright August 1989 - 1999 by Billy Chan and Malcolm Bibby. All

rights reserved. The software described in this document is furnished under a

License Agreement.

FOREWORD

This system is specially designed as an aid for welding engineers who

must make welding parameters specifications (current, voltage, welding speed,

preheat, postheat etc.) but are too busy to go through lengthy calculations or

written documentation. The software is easy to start and user-friendly. If you are a

hands-on adventurer, turn on the computer and follow the 'Start Up' and the 'Quick

Tutorial' sections of this manual or if you are a reasonably experienced computer

user and are experienced in welding technology, simply type "weldsoft" after the

DOS prompt and follow the screen directions to work through the system

intuitively. You will be able to master the system within 30 minutes. However, a

complete user's manual is provided for your reference. Valuable background

material is covered in this manual. Please spend some time to read it !!

TABLE OF CONTENTS

LIMITED WARRANTY	i

LICENSE AGREEMENT	i

FOREWORD	ii

TABLE OF CONTENTS	iii

SYMBOLS	iv

Chapter 1		Introduction	1

Chapter 2		Start Up	3

2.1	System Requirements	3

2.2	Installing WeldSoft	4

Chapter 3		Quick Tutorial	6

3.1	Loading WeldSoft	6

3.2	Changing Default Set Up	7

3.3	Calculating HAZ Hardness	8

3.4	Data File Features	10

3.5	Plotting a Graph	12

3.6	Output to a Printer	13

3.7	Changing the Default Settings inside the

Module	14

3.8	On-Line Help	14

3.9	Exiting HAZ Hardness Module	15

3.10	Next Move	15

Chapter 4		Operation Guide	17

4.1	Documentation Section	18

4.2	Set Up Section	20

4.3	HAZ Hardness Module	23

4.4	Reheat Hardness Module	31

4.5	Preheat Treatment Module	37

Chapter 5		Background Material	46

5.1	HAZ Hardness Module	46

5.2	Reheat Hardness Module	48

5.3	Preheat Treatment Module	49

SYMBOLS

<F1>			the F1 function key

<F2>			the F2 function key

<F3>			the F3 function key

<F4>			the F4 function key

<F5>			the F5 function key

<F6>			the F6 function key

<F7>			the F7 function key

<F8>			the F8 function key

<F9>			the F9 function key

<ESC>			the ESCAPE key

<←>/<→>   		the Left / Right arrow keys

<↑>/<↓>		the Up / Down arrow keys

<PgUp>/<PgDn>	the PgUp / PgDn keys

<enter>		the carriage return or ENTER key

'*****'			function box identification

{*****}		comment only

Chapter 1     Introduction

Welcome to WeldSoft, a software system for busy welding engineers.

The system is designed as a computer aid for professionals involved with

welded fabrication - welding engineers, consultants or specialists associated with

large fabrication facilities, mechanical design engineers or material engineers. It is

often necessary for engineers to specify welding parameters and conditions for

fabrication specifications. Traditionally, specifications have been based on either

long experience or on trial run activities. This software provides some guidance

where the criteria involves heat-affected zone (HAZ) hardness, HAZ hardness

after post weld heat treatment (reheat hardness) or preheat temperature to avoid

under-bead cracking.

Users with reasonable background in welding engineering and a basic

knowledge in personal computing can easily use the system. It is menu (function

keys) and mouse (optional) driven with the following features:

?	Calculate/Re-calculate: with all the necessary welding conditions and/or

base metal chemical composition the HAZ hardness, HAZ hardness after

post weld heat treatment or minimum preheat temperature to avoid under-

bead cold cracking can be predicted with a touch on the keyboard, <F1>,

or a click of the mouse button.

?	Pop-up / Pull-down menu: all complicated WeldSoft features can be

accessed through pop-up/pull-down menus that temporarily overlay what

the user is doing. The menus will guide the user through step-by-step

without hesitation or referring back to the user-manual.

?	On-Line Help: help window, <F9>, describes the features of the current

module.

?	Retrieve / Store Data File: frequently used data (welding parameters or

steel chemical composition) can be stored in or retrieved from data files,

<F7>. Moreover, existing data files can be edited.

?	Plot Graph (Parametric Study): automatic generation of graphs, <F6>, is a

special feature of this system. After x-axis and y-axis (where applicable)

are selected and the range of parametric study is set through a pop-up

menu, a graphics quality graph is automatically plotted with titles, scales

and legends.

?	Print Screen: graphics quality hard-copy is available, <F8>, if the

computer is connected to an either Epson dot matrix or HP II Laser Jet

printer (or compatible).

?	Easy to Quit: you will never feel that you are trapped in this environment.

To quit, it is as easy as selecting 'EXIT' or <ESC> where applicable.

There are some special features specific to each individual module. They

will be introduced and discussed where appropriate later in this manual.

All modules are based on regression models and therefore, they are subject

to certain working limits (listed in the next few chapters). In general, the

mathematical relationships are valid for low alloy steels with carbon content less

than 0.3 wt%. However, regression limits are not strictly imposed in the

calculations since it is often interesting to explore beyond the limits. On the other

hand, limits can be imposed by selecting the "limits active" option to ensure that

they are not violated. It is left to the user to use the software wisely !!

Chapter 2     Start Up

The WeldSoft package contains the following items; please check before

proceeding :

?	user manual ;

?	a 32" or 53" floppy disk which contains the following files :

-  WELDSOFT.EXE (or WELD2.EXE);

-  WELDSOFT.OVR (or WELD2.OVR);

-  *.HAZ, *.REH, *.PRE & *.STE ;

-  HDInstal.BAT ;

IMPORTANT NOTE

Before starting, make a backup copy of the original disk. In case of

corrupted data file(s), the original file(s) can be used to replace the bad file(s). To

make a backup copy, follow these instructions :

-	put the write-protected original disk into drive A;

-	at DOS prompt type :-  diskcopy a: a: <enter>

Note : if drive B is used, type :- diskcopy b: b: <enter>

-	follow the instruction displaced on screen ;

After a backup copy is made, put away the original disk in a safe place and

use the backup copy as the working disk.

2.1   System Requirements

WeldSoft requires an IBM-PC, PC/XT, PC/AT or compatible (an Intel

286 or 386 processor is highly recommended), with the following equipment:

?	512k RAM minimum ;

?	MS-DOS or compatible version 3.3 or later ;

?	one floppy disk drive (a hard disk system is strongly recommended) ;

?	a high resolution monitor (at least 640x348 active pixels) ;

Optional equipment :

?	mouse (Microsoft or Logitech compatible) ; and

?	Epson dot matrix printer (or compatible), or HP II Laser Jet (or PCL

compatible) ;

Note :	a) If the floppy disk drive is the only source, the floppy disk should not be

write-protected.

b) If a mouse is present, please install the mouse driver before starting the

program, i.e. type "mouse" after the DOS prompt.

c) The printer should be 'on-line' for printing.

2.2   Installing WeldSoft

There is almost no installation required before starting WeldSoft.

However, good hard disk management practices should be followed.

Using a Floppy Drive System

If you are using a floppy drive system without a hard disk, installation is

not required. To use the 'store or edit files' features in the system, the floppy disk

should not be write-protected.

Using a Hard Disk System

If a hard disk is used, a sub-directory for holding all the related files is a

good idea. To install the system to the hard disk (drive C) sub-directory called

'Weld', simply follow the instructions:

-	put the working disk into drive A (or B) ;

-	change the log drive to the drive holding the working disk by typing :- a:

<enter> or (b:)

-	type :- HDInstal <enter>

Sit back and relax. The install program creates a sub-directory 'WELD' in

drive C. After all files have been copied from the working disk to the sub-

directory 'WELD', the system starts up automatically.

If you want to install the system to a different sub-directory, put the

working disk into Drive A and type the following commands at C prompt:

C:\>md sub-directory_name <enter>

C:\>copy a:*.* c:\sub-directory_name <enter>

The drive destination 'a:' can be changed if the working disk is in another drive.